Contents

Generating DTBO file: -	2

VVI Link	2

Raspberry pi camera	2

1. The Big Picture: What Linux Needs	3

2. Step-by-Step Implementation Flow	3

Step A: Generate the .bin File	3

Step B: Generate the Device Tree Overlay (.dtbo)	3

Step C: Create the shell.json	4

Step D: Load it on the KV260 using xmutil	4

3. Essential Learning Resources & Tutorials	4

Yocto sync and build	5

Step 1: Pre-requisites & Dependencies	5

Step 2: Initialize & Sync the Yocto Code Base	5

Step 3: Inject Your Custom FPGA Bitstream (.xsa)	5

Step 4: Build the Image & Flash to SD Card	6

Boot Linux Getting Started	6

https://xilinx.github.io/kria-apps-docs/kv260/2021.1/linux_boot/petalinux_2021.1/build/html/docs/sdcard.html	6

KV260 Tutorials	6

Generate custom firmware	7

Setting up Yocto environment	7

Updating default bit stream	7

Vitis  tutorial	7

Function Acceleration on FPGA with Vitis-Part 1: Fundamental  Udemy	7

Embedded System Design with Xilinx Zynq SoC and Vitis IDE = > Udemy	7

## Generating DTBO file: -

https://xilinx.github.io/kria-apps-docs/creating_applications/2022.1/build/html/docs/dtsi_dtbo_generation.html

https://github.com/Xilinx/kria-apps-firmware/tree/main/boards

https://xilinx-wiki.atlassian.net/wiki/spaces/A/pages/18842279/Build+Device+Tree+Blob

https://www.hackster.io/LogicTronix/kv260-bist-tutorial-recreating-with-vivado-vitis-2025-1-09f010

https://adaptivesupport.amd.com/s/question/0D54U00007mDJ6ZSAW/how-to-use-custom-bitstream-in-ubuntu-2204?language=en_US

https://www.udemy.com/course/learn-zynq-ultrascale-plus-mpsoc-development/?srsltid=AU7gw4VRvRvmHcTYKBxb1eAdjv5C8jIU52omzPXhZbQhMYdt8SFAdxYX&couponCode=KEEPLEARNING

https://adaptivesupport.amd.com/s/question/0D5Pd00001NOHTbKAP/kria-kv260-yocto-build-with-custom-pl?language=en_US

https://community.element14.com/products/roadtest/b/blog/posts/amd-xilinx-kria-kv260-vision-ai-starter-kit-quantizing-and-compiling-a-tensorflow-pytorch-model-for-kv260

## VVI Link

https://github.com/Xilinx/Xilinx_Kria_KV260_Workshop/blob/main/Part%204%3A%20Optional%20Demo.md

https://xilinx.github.io/kria-apps-docs/kv260/2022.1/build/html/docs/nlp-smartvision/docs/app_deployment_nlp.html#downloading-and-loading-the-application-firmware

## Raspberry pi camera

https://xilinx.github.io/kria-apps-docs/kv260/2022.1/build/html/docs/rpi-v3/run.html

https://xilinx.github.io/kria-apps-docs/creating_applications/2022.1/build/html/docs/vitis_platform_flow_smartcam_raspi_example.html

https://xilinx.github.io/kria-apps-docs/kv260/2022.1/build/html/docs/smartcamera/smartcamera_landing.html

## AI flow

Welcome to the world of FPGA! Coming from an Embedded Linux background, you already know half of the equation (kernels, rootfs, and device trees). The main paradigm shift with AMD Kria devices (like the Kria KV260) is that the programmable logic (PL) is treated as a dynamic, hot-pluggable peripheral using Linux's FPGA Manager via a tool called xmutil.

You do not need to reboot Linux to flash your FPGA; you load it at runtime as a firmware package.

Here is the exact step-by-step workflow and the best documentation links to learn this process.

## 1. The Big Picture: What Linux Needs

To load a custom bitstream on Kria Ubuntu or PetaLinux, you must package your Vivado design into a Firmware Directory containing three specific files: [1, 2]

- name.bin: The actual FPGA bitstream (binary format).

- name.dtbo: A Device Tree Blob Overlay that tells the Linux kernel what drivers to load for your custom PL hardware blocks.

- shell.json: A simple configuration file mapping the slots. [1, 2]

All three files must share the exact same base name and be placed in /lib/firmware/xilinx/name/ on your KV260 target. [2]

## 2. Step-by-Step Implementation Flow

## Step A: Generate the .bin File

When you complete your hardware design in Vivado, it outputs a standard .bit file. Linux needs a binary format. [1]

- You can easily convert this inside the Vivado/Vitis XSCT console using standard bootgen commands, or simply isolate the raw bitstream from your exported .xsa file and rename it to a .bin extension. [1, 2]

## Step B: Generate the Device Tree Overlay (.dtbo)

Because you are changing hardware at runtime, the Linux kernel needs to know what just appeared in memory.

- Use the AMD Xilinx device-tree-compiler (DTC) along with their specialized hardware flow to target your .xsa file. This generates a .dtsi source file which you compile into a .dtbo binary object. [1, 2]

## Step C: Create the shell.json

This metadata file instructs the Kria resource manager. You can copy a reference template directly from the AMD Kria Apps Firmware GitHub Repository. [1]

## Step D: Load it on the KV260 using xmutil

Once you transfer your directory to /lib/firmware/xilinx/your_app/ on the board via SSH or an SD card, use the Kria-specific utilities to swap it dynamically: [2]

# List available firmware overlays (yours should appear here)

sudo xmutil listapps

# Unload the default/current bitstream application

sudo xmutil unloadapp

# Load your custom bitstream application

sudo xmutil loadapp your_app

## 3. Essential Learning Resources & Tutorials

- The Definitive Practical Guide: Read the excellent community tutorial on Custom PL Designs on Kria KV260 on Hackster.io. This walkthrough covers utilizing AMD's Embedded Development Framework (EDF), generating the correct device trees, and loading the bitstream live. [3]

- Official Workflow Documentation: Read the official AMD Kria Apps Device Tree Generation Guide to understand the XSCT compiler approach for building the required hardware-to-Linux bridge. [1]

- Troubleshooting & Community QA: For a quick sanity check on file naming conventions and directory structures for Ubuntu environments, bookmark this AMD Adaptive Support Kria Ubuntu Custom Bitstream Thread. [2]

Are you planning to build your Embedded Linux environment using Ubuntu for Kria or are you building a completely custom distribution using PetaLinux/Yocto? Let me know, and I can give you the exact device tree compilation snippet for either environment!

[1] https://adaptivesupport.amd.com

[2] https://adaptivesupport.amd.com

[3] https://www.hackster.io

## Yocto sync and build

Yes, if you want to bake your custom FPGA bitstream directly into the bootable Linux system image from scratch, you must sync the AMD/Xilinx Yocto code base. This will allow BitBake to build a ready-to-flash .wic image that contains your system files and custom hardware configuration right out of the box. [1, 2, 3, 4]

Alternatively, if you use PetaLinux (which is AMD’s packaged Yocto wrapper), you do not need to manually handle repo sync. However, if you are doing pure Yocto development, the setup process is detailed below. [1, 2, 5]

## Step 1: Pre-requisites & Dependencies

Ensure your Linux host has the required Yocto packages and Google's repo tool installed: [5, 6]

# Install repo tool

mkdir -p ~/bin

curl https://storage.googleapis.com/git-repo-downloads/repo > ~/bin/repo

chmod a+x ~/bin/repo

export PATH=$PATH:~/bin

## Step 2: Initialize & Sync the Yocto Code Base

You will use the AMD Yocto Manifests Repository to track and synchronize all the custom meta-layers (such as meta-xilinx, meta-kria, and meta-petalinux): [1]

# Create and move into your build workspace

mkdir -p ~/kria-yocto-bsp && cd ~/kria-yocto-bsp

# Initialize the repository targeting your preferred release version (e.g., rel-v2025.1)

repo init -u https://github.com/Xilinx/yocto-manifests.git -b rel-v2025.1

# Sync all meta-layers (This downloads a significant amount of source data)

repo sync

# Start a development branch tracking the release

repo start rel-v2025.1 --all

## Step 3: Inject Your Custom FPGA Bitstream (.xsa)

To embed your custom hardware design inside the Yocto build structure: [3]

- Set up the build workspace environment variables using the provided initialization script:

- source setupsdk

- Open your local configuration file conf/local.conf inside your build directory.

- Add the path variables instructing Yocto to look for your exported Vivado hardware handoff file (.xsa), which implicitly includes your bitstream: [1, 3, 7, 8]

- HDF_BASE = "file://"

- HDF_PATH = "/absolute/path/to/your_custom_design.xsa"

## Step 4: Build the Image & Flash to SD Card

Target the universal machine profile configured for Kria starter kits to compile the build: [4, 8]

# Compile and generate the complete rootfs and WIC image

MACHINE=kria-zynqmp-generic bitbake kria-image-full-cmdline

Once completed, the deployment folder (tmp/deploy/images/kria-zynqmp-generic/) will output a .wic file. Flash this raw image file directly to your SD card using utility tools like Balena Etcher or the dd terminal utility. [3, 4]

Would you prefer to proceed with this pure Yocto manifest flow, or would you like to see how to accomplish this using PetaLinux BSP project commands, which can simplify target bitstream integration? Let me know your preference.

[1] https://github.com

[2] https://www.hackster.io

[3] https://adaptivesupport.amd.com

[4] https://www.hackster.io

[5] https://xilinx-wiki.atlassian.net

[6] https://xilinx-wiki.atlassian.net

[7] https://xilinx.github.io

[8] https://xilinx.github.io

## Boot Linux Getting Started

https://xilinx.github.io/kria-apps-docs/kv260/2021.1/linux_boot/petalinux_2021.1/build/html/docs/intro.html

## https://xilinx.github.io/kria-apps-docs/kv260/2021.1/linux_boot/petalinux_2021.1/build/html/docs/sdcard.html

## KV260 Tutorials

https://xilinx.github.io/kria-apps-docs/kv260/2022.1/build/html/docs/linux_boot.html

## Generate custom firmware

https://xilinx.github.io/kria-apps-docs/kv260/2022.1/build/html/docs/generating_custom_firmware.html

## Setting up Yocto environment

https://edf.docs.amd.com/en/latest/yocto/setting-up-the-yocto-environment.html

## Updating default bit stream

https://xilinx.github.io/kria-apps-docs/yocto/build/html/docs/yocto_kria_support.html#updating-default-bitstream

## Vitis  tutorial

https://github.com/xilinx/Vitis-Tutorials

# Function Acceleration on FPGA with Vitis-Part 1: Fundamental  Udemy

# Embedded System Design with Xilinx Zynq SoC and Vitis IDE = > Udemy